## 4.5 Biopolymers: Lignin Degradation as Process-Driven Transition

A fourth domain extends the framework beyond time-series trajectories to cross-sectional process data. Kraft pulping of lignocellulosic biomass subjects lignin—a heterogeneous biopolymer network—to controlled chemical degradation via three process parameters: severity factor (P-factor), temperature, and liquid-to-solid ratio. The resulting structural and physicochemical properties (inter-monomer linkage density β-O-4, average chain length, glass transition temperature Tg, reactivity index RSI, surface tension at multiple concentrations) represent measurable order parameters of the degradation state.

Data from n = 90 Kraft-pulped lignin samples (58–73 with complete analytical characterization depending on output variable) were analyzed. Process inputs were normalized to [0, 1] via min-max scaling and combined into a dimensionless load parameter Σ. For each structural output Y, a normalized order parameter Q ∈ [0, 1] was constructed via min-max scaling with orientation correction (ensuring Q increases with Σ). Binned means (8–10 bins, ≥3 samples per bin) were fitted with the sigmoid transition model:

where â estimates the inflection point (critical process severity) and k the transition steepness.

### Results

The strongest monotone association was observed for β-O-4 linkage density as a function of P-factor (ρS = −0.77, n = 90), with sigmoid fit parameters â = 0.325 and k = 2.86 (RMSEbinned = 0.083). Surface tension at 0.5 mg/ml showed ρS = 0.61 with â = 0.21; RSI showed ρS = 0.59 with â = 0.08. Across all non-degenerate fits (n = 7 output–load combinations with 0.05 < â < 0.95), the critical inflection point clustered at:

This value is markedly lower than the semantic domain (Σc ≈ 0.6) and the materials domain (where â corresponds to the normalized yield strain, typically 0.1–0.3 for wood). The form of the transition is identical; the location is domain-characteristic.

Table 3. Sigmoid fits for strongest process–property associations (Σ = P-factor normalized)

| Output Y | n | ρS | â | k | RMSE | Orientation |
| --- | --- | --- | --- | --- | --- | --- |
| β-O-4 linkage density | 90 | −0.77 | 0.325 | 2.86 | 0.083 | decreasing |
| Surface tension (0.5) | 58 | 0.61 | 0.210 | 1.94 | 0.094 | increasing |
| RSI | 73 | 0.59 | 0.080 | 1.66 | 0.058 | increasing |
| Surface tension (0.25) | 58 | 0.55 | 0.334 | 1.52 | 0.105 | increasing |
| Chain length | 90 | −0.55 | * | 1.82 | 0.088 | decreasing |
| Tg (avg3) | 58 | 0.38 | 0.344 | 2.42 | 0.068 | increasing |

Note: * denotes degenerate fit (â ≈ 0 or ≈ 1), excluded from critical-point aggregation. Orientation indicates raw Spearman direction before Q-normalization.

### Interpretation Within the SRDS Framework

The lignin system maps onto the axiomatic structure as follows. The state zt is the ensemble of molecular configurations (linkage types, chain lengths, functional group distribution) at a given process severity. The operator Ft encodes the kinetic degradation regime: below the critical severity, bond cleavage is selective and structure-preserving (analogous to elastic deformation); above it, non-selective fragmentation and condensation reactions dominate (analogous to plastic flow). The transition Fselective → Fnon-selective is an operator transition in the sense of Axiom A0.

The critical severity â marks the inflection where order parameters undergo their most rapid change—the biopolymer analogue of a yield point. The sigmoid functional form Q(Σ) = 1/(1 + exp(−k(Σ − â))) captures the transition from intact to degraded regime, with k encoding the sharpness of the transition (higher k corresponding to a more abrupt, brittle-like failure).

### Domain-Specificity of the Critical Threshold

A key observation: the critical inflection â is not universal across domains. In the semantic domain, the stability metric Σc ≈ 0.6 was empirically identified as a threshold for system instability (Unifying ENK/SKB/MI Formalization, Scheßl 2025). In the materials domain, the normalized yield strain is typically 0.1–0.3. In the biopolymer domain, â ≈ 0.28–0.33 for the dominant structural outputs.

This is consistent with the theoretical expectation: the functional form Q(Σ) = 1/(1 + exp(−k(Σ − â))) is structurally invariant—a consequence of the universal grammar defined by Axioms A0–A4. The location â is set by domain-specific physics: bond energies and kinetic barriers in materials, attention dynamics and token probability distributions in semantic systems, reaction selectivity thresholds in biopolymer chemistry. A universal constant Σc would, in fact, contradict the framework, since the axioms prescribe transition structure (A4), not transition location.

Table 4. Extended Unified Axiom Mapping (four domains)

| Axiom | Materials (Yield) | Vision (Canon.) | Semantics (LLM) | Biopolymer (Lignin) |
| --- | --- | --- | --- | --- |
| A0 | Elastic→Plastic law shift | Retrain on self-transformed data | Context updates constrain next turn | Selective→Non-selective degradation regime shift |
| A1 | Heat export maintains structure | Regularization bounds variance | Attention suppresses entropy | Black liquor export removes degradation products |
| A2 | C = U − TS | C = classification loss | C = divergence / instability | C = residual linkage integrity |
| A3 | Q = stress–strain slope | Q = pose manifold | Q = VE, FVI, RPM, DIV | Q = β-O-4, chain length, Tg, RSI |
| A4 | λmax = 0 at yield | Alignment collapse | Narrative Yield Point | Sigmoid inflection at â ≈ 0.3 |

Table 5. Extended Structural Isomorphism (four domains)

| Component | Materials | Vision | Semantics | Biopolymer |
| --- | --- | --- | --- | --- |
| State z | Microstructure | Embeddings | Embeddings + context | Molecular configuration ensemble |
| Operator F | Constitutive law | Canon. + classifier | Effective policy | Kinetic degradation regime |
| Order Param. Q | Stress–strain slope | Pose manifold | VE, RPM | β-O-4, chain length, Tg |
| Transition criterion | ε ≥ εy | Alignment loss plateau | ΔVE ≥ 0.4·VEbase | Σ ≥ â ≈ 0.3 (P-factor) |
| Data type | Time series (190K pts) | Training iterations | Turn sequence (714 turns) | Cross-section (n = 90) |
| â range | 0.1–0.3 (norm.) | — | ≈ 0.6 | 0.28 ± 0.13 |

### Limitations and Required Extensions

Three limitations qualify this fourth domain projection. First, the lignin data are cross-sectional: each sample represents a different set of process conditions, not a single system observed through a continuous transition. The sigmoid describes the ensemble response surface, not an individual trajectory. The other three domains provide trajectory data (load–displacement curves, training iterations, conversation turns). Extension to raw DSC thermograms—where a single specimen traverses the glass transition as a function of temperature—would close this gap and provide the biopolymer equivalent of a stress–strain curve.

Second, sigmoid functions are flexible two-parameter models that fit any monotone-saturating relationship. The goodness of fit (RMSE ≈ 0.06–0.10 on binned means) is necessary but not sufficient evidence for the axiomatic structure; it establishes functional form consistency without confirming operator dynamics. Independent validation of the Fselective → Fnon-selective transition would require mechanistic kinetic modeling.

Third, the sample size (n = 58–90 depending on output) provides adequate statistical power for detecting monotone trends (ρS > 0.5 significant at p < 0.001) but limited resolution for distinguishing sigmoid from other saturating forms (e.g., Hill equation, Weibull CDF). Larger datasets with denser sampling near the inflection region would improve discrimination.

### Synthesis

Despite these qualifications, the biopolymer data provide a fourth independent consistency check for the SRDS framework. The central empirical finding is not a universal critical constant, but a universal transition form: across four substrates—crystalline wood under mechanical load, neural network training dynamics, human–LLM semantic interaction, and biopolymer degradation under chemical processing—the order parameter Q collapses sigmoidally as a function of normalized load Σ, with domain-characteristic inflection â and steepness k. This is the empirical signature of Axiom A4: critical transitions occur via order-parameter discontinuities whose structure is universal while whose location is set by substrate physics. The framework predicts that any self-referential dissipative system will exhibit this form; each new domain that confirms it narrows the space of alternative explanations.